MEMORIAL HALL

Technical Specifications

1225 Elm St, Cincinnati, OH 45202   ·   memorialhallotr.com   ·   Virtual tour: memorialhallotr.com/virtual-tour

| DIRECTOR OF MEMORIAL HALL Joshua Steele JSteele@3cdc.org | SENIOR PRODUCTION SUPERVISOR Brian Lloyd BLloyd@3cdc.org |
| --- | --- |

# AT A GLANCE

| Capacity | 556 seats: 215 on the main level (orchestra) and 341 in the balcony. All 11 accessible seats are on the main level. |
| --- | --- |
| Stage | Trapezoidal, 37'4" at the widest point and 22'5" deep at center. Permanent orchestra shell; no soft goods and no overstage rigging. |
| Load-in | Street-level door on Grant Street (2'7" W × 6'10" H) into a freight elevator. No dock. The stage is on the second floor. |
| Power | One 400A 3-phase disconnect, backstage stage right. No shore power; generators are OK. |
| House console | DiGiCo Quantum 225 with Pulse and a DQ Stage rack (48 inputs), in the house right balcony. |
| PA | Sound Bridge line array (main and balcony hangs, L/R), dual 18" subs and 4 front fills. |
| Monitors | 8 wedges and 4 in-ear systems, mixed from FOH. A Yamaha CL3 monitor console and split are available; a fee may apply. |
| Lighting | ETC Ion Element 40 running an LED rep rig with Source 4s, 8 moving heads and 4 truss towers. |
| Video | 10,000-lumen laser projector and a 20' motorized 16:9 screen, fed by HDMI from backstage. |
| Sound limit | 95 dB LAeq,6min (A-weighted, 6-minute average) at balcony center, per local ordinance. |
| Building curfew | 90 minutes after the scheduled end of the show. All touring personnel, artists and gear must be out of the building by then. |

The room from the stage, with the house FOH, house lighting, touring lighting and touring FOH positions marked

# STAGE

| Shape | Trapezoidal. |
| --- | --- |
| Width | 22'5" at the back wall; 37'4" at the widest point. |
| Depth | 22'5" at the deepest point (center stage). |
| Height | 11'5" at the back-wall ledge; 18' at center stage; 23' at downstage center. |
| Shell | Permanent orchestra shell. The ornate upstage wall is 12' high and scallops to 20'+ at the plaster line. |
| Rigging | No soft goods. No overstage rigging for backdrops, soft goods or lighting. |
| Entrances | Stage left and stage right doorways, 2'7" W × 6'10" H. A 5'11" piano door on the upstage right wall for load-in. |
| House to stage | Steps from the house to the stage, with or without a railing, can be added in the aisles. |
| Building | Opened 1908. Listed on the National Register of Historic Places in 1978. |

Stage plan with dimensions

| The stage from the orchestra | Trim and ledge heights on the upstage wall |
| --- | --- |

# LOAD-IN, PARKING & POWER

| Loading door | On Grant Street at street level. Standard doorway, 2'7" W × 6'10" H. Flat push; no loading dock. |
| --- | --- |
| Freight elevator | Just inside the loading door. 5' W × 7'8" L × 7' H. |
| Stage level | The stage is on the second floor. |
| Parking | Details come with your advance. |
| Power | One 400A 3-phase disconnect, backstage stage right. |
| Shore power | None. Generators are OK. |

| Loading door on Grant Street | Freight elevator, just inside the loading door | Inside the freight elevator |
| --- | --- | --- |

# AUDIO

## PA SYSTEM

| QTY | MODEL | POSITION |
| --- | --- | --- |
| 3 per hang | Sound Bridge Xyon 7108XY line array | Main and balcony hangs, L/R |
| 2 per side | Sound Bridge 7218SWX dual 18" subwoofer | Ground-stacked, L/R |
| 4 | Sound Bridge 3208HTH front fill | Mono |

## FRONT OF HOUSE

| House console | DiGiCo Quantum 225 with Pulse. |
| --- | --- |
| Stage rack | DiGiCo DQ Stage rack, 48 inputs. |
| Position | House right balcony, with a full view of the stage and no booth or overhang. The house console is in a fixed position and does not move. |

| House FOH position, house right balcony (red box) | View from the house FOH console |
| --- | --- |

## TOURING FOH

| Routing | Guest consoles run through the house console. We can accept left, right, sub, fill and assisted listening feeds, or any combination of them. |
| --- | --- |
| Position | Rear center of the orchestra (Box M), with a full view of the stage and no booth or overhang. |
| Cabling | No drylines at this position; PA drivelines are stage left. The run to the stage box is about 150'. Tours must bring all cables needed to connect the touring console to any stage racks or devices. |

| Touring FOH position, Box M (red box) | View from the touring FOH position |
| --- | --- |

## MONITORS

| Wedges | 8 Sound Bridge 7122W 12" stage monitors. |
| --- | --- |
| In-ears | 4 Sennheiser EW300 in-ear systems. |
| Monitor mixes | Run from FOH on the house console unless a monitor console is in use. |
| Monitor console | Yamaha CL3, 48 inputs. A fee may apply. |
| Split | 40-channel transformer-isolated split. It's required when using the house monitor console, and it can also feed a touring monitor console. A fee may apply. |
| Monitor position | Offstage left, through a 2'7" W × 6'10" H doorway. |

| Monitor position, offstage left | View from the monitor position |
| --- | --- |

## SOUND LEVEL LIMIT

Local ordinance limits Memorial Hall to 95 dB LAeq,6min: the A-weighted equivalent level averaged over 6 minutes, measured at balcony center.

## RECORDING & ARCHIVING

We offer recording and archiving packages. Package options are available on request.

## MICROPHONES & DI

| QTY | MODEL | TYPE | NOTE |
| --- | --- | --- | --- |
| 8 | Shure SM58 | Dynamic, vocal |  |
| 8 | Shure SM57 | Dynamic, instrument |  |
| 1 | Shure Beta 57A | Dynamic, instrument |  |
| 1 | Shure Beta 52A | Dynamic, kick |  |
| 1 | Shure Beta 91A | Boundary, kick |  |
| 2 | Sennheiser e609 | Dynamic, guitar cab |  |
| 4 | Sennheiser e604 | Dynamic, drum clip |  |
| 2 | Shure SM81 | Small-diaphragm condenser |  |
| 2 | Shure Beta 27 | Large-diaphragm condenser |  |
| 2 | AKG C414 | Large-diaphragm condenser |  |
| 2 | Telefunken M60 | Small-diaphragm condenser |  |
| 4 | Shure ULX-D2 / Beta 58A | Wireless handheld |  |
| 2 | Shure SLX / SM58 | Wireless handheld |  |
| 3 | Lavalier mic | Lavalier | May incur a fee |
| 5 | Countryman E6 | Headset | May incur a fee |
| 10 | Radial JDI | Passive DI |  |

Specific mic requests are possible with advance notice, but additional fees may apply.

# LIGHTING

| House console | ETC Ion Element 40, at the rear of the upper balcony, center. |
| --- | --- |
| House lights | Our console and crew always run the house lights. |
| Tour console | Touring lighting consoles go in the house left balcony. |
| Ground package | Tours bringing a ground package must supply 150' of cable to reach it. |
| Paperwork | The house rep plot and other lighting paperwork are available on request. |

## INVENTORY

| QTY | FIXTURE | NOTE |
| --- | --- | --- |
| 14 | ETC Source 4, 14° and 19° |  |
| 10 | ETC Lustre LED, zoom |  |
| 2 | Robin DLX moving head |  |
| 6 | LED wash/zoom moving head, 36 × 18W |  |
| 14 | LED overhead track wash |  |
| 8 | LED uplight |  |
| 4 | Elation Fuze 120 LED wash |  |
| 2 | 1 × 1 × 5 stand-alone truss, 4 LED flat pars each | Mounted |
| 2 | 1 × 1 × 8 stand-alone truss, 4 LED flat pars each | Mounted |
| 1 | Radiant water-based hazer | Additional fee |
| 2 | Spotlight | Additional fee |

| View from the house lighting console | Touring lighting position in the house left balcony (red box), shot from house right | View from the touring lighting position |
| --- | --- | --- |

# VIDEO

| Projector | Panasonic PT-RZ970BU laser projector, 10,000 lumens, WUXGA. |
| --- | --- |
| Screen | 20' motorized screen, 135" × 240", 16:9. It can be partly lowered, with the picture resized to fit. |
| Position | About 13' from the back wall and 9' upstage of the lip. The screen can't be rehung. |
| Input | HDMI from backstage. |

| The screen fully lowered, seen from the orchestra | The screen from backstage, stage left |
| --- | --- |

# BACKLINE & STAGE EQUIPMENT

The house has no backline instruments or amps, but we can arrange backline rentals.

## PIANO

Yamaha C3 PE grand (6'1") with Helpinstill pickups.

## STANDS & SEATING

| QTY | ITEM | NOTE |
| --- | --- | --- |
| 10 | Tall tripod boom stand |  |
| 2 | Medium tripod boom stand |  |
| 6 | Short tripod boom stand |  |
| 2 | Tall round-base boom stand |  |
| 5 | Tall round-base straight stand |  |
| 4 | Short tabletop mic stand |  |
| 11 | Guitar stand |  |
| 25 | Music stand |  |
| 1 | Conductor's music stand |  |
| 25 | Clip light | May incur a fee |
| 20 | Wenger orchestra chair |  |
| 6 | Backless barstool, unpadded |  |

## RISERS & STAGING

| QTY | ITEM | NOTE |
| --- | --- | --- |
| 4 | Wenger riser, 4' × 8' × 12" | Black skirting |
| 6 | Wenger 3-level choral riser |  |
| 1 | Conductor's podium |  |
|  | Pipe and drape | Covers the back wall of the stage |
| 1 | Large wooden podium | Gooseneck mic |
| 1 | Small black podium | Gooseneck mic |
|  | Drum rugs |  |

# DRESSING ROOMS & AMENITIES

Five dressing rooms, all lockable and climate-controlled.

| ROOM | LEVEL | CAPACITY | FEATURES |
| --- | --- | --- | --- |
| A | Stage level, stage right | 2–3 | Vanity mirror and counter; private toilet |
| B | Stage level, stage left | 2–3 | Vanity mirror and counter; private toilet (identical to A) |
| C | Balcony level, stage right | 5 | Mirrors, tables, chairs and couches; no private toilet |
| D | Balcony level, stage left | 5 | Mirrors, tables, chairs and couches; no private toilet |
| Studio | Basement | Large group | Multipurpose room with mirrors, tables and chairs; restroom shared with venue staff |

Black stage towels are available. There are no showers or laundry facilities on site.

| Stage-level dressing room (A and B match) | Stage-level dressing room, vanity and counter | The Studio, basement level |
| --- | --- | --- |

| Dressing Room C, balcony level, stage right | Dressing Room D, balcony level, stage left |
| --- | --- |

# ADDITIONAL TERMS

| Building curfew | 90 minutes after the scheduled end of the show. All touring personnel, artists and gear must be out of the building by then. |
| --- | --- |
| Left behind | Tell us right away if you leave something behind. Anything left in the building may be thrown out. |
| Radius | No other shows within 90 miles of Cincinnati for 60 days before or after your date. |
| Merchandise | The house takes no cut of merch sales. Bring your own POS system and bank. We can help arrange a seller if you need one. |
| Security | We may have security on site, but you're responsible for your own gear and belongings. |